Flow Analysis of NACA 0012 Airfoil Using Discontinuous Galerkin Spectral Element Method: Viscous Flow Study

# Abstract

This paper presents a comprehensive numerical study of flow around the NACA 0012 airfoil using the Discontinuous Galerkin Spectral Element Method (DGSEM). The analysis focuses on viscous flow computations using the two-dimensional Navier-Stokes equations at various Reynolds numbers ranging from 10³ to 10⁶. The DGSEM implementation employs polynomial degree N = 3 with Legendre-Gauss-Lobatto quadrature points and Roe numerical flux for accurate capture of flow physics. Validation is performed against established experimental data from Gregory and O'Reilly (1970), with cross-verification using OpenFOAM simulations. The results demonstrate the effectiveness of high-order DGSEM for capturing complex flow features including boundary layer development, flow separation, and wake characteristics.

# 1  Introduction

The NACA 0012 airfoil represents one of the most extensively studied geometries in computational fluid dynamics and experimental aerodynamics. Its symmetric profile and well-documented flow characteristics make it an ideal benchmark for validating numerical methods and understanding fundamental aerodynamic phenomena. The airfoil's behavior across different Reynolds number regimes provides valuable insights into the transition from laminar to turbulent flow and the associated changes in separation patterns and aerodynamic performance.

The Reynolds number, defined as Re = ρU∞L/μ, plays a crucial role in determining flow behavior around airfoils, influencing boundary layer development, flow separation, and wake characteristics. At low Reynolds numbers (Re < 10⁴), the flow typically remains attached with laminar separation occurring near the trailing edge. As the Reynolds number increases, the separation point moves forward, and the flow becomes increasingly complex with potential for turbulent transition and reattachment. Gregory and O'Reilly (1970) provided comprehensive experimental data for NACA 0012 at Reynolds numbers of 2.88 × 10⁶ and 1.44 × 10⁶, which serves as a valuable benchmark for computational validation.

The Discontinuous Galerkin Spectral Element Method (DGSEM) has emerged as a powerful computational approach for solving conservation laws, offering high-order accuracy and excellent parallel scalability. Unlike traditional finite difference or finite volume methods, DGSEM combines the geometric flexibility of finite elements with the spectral accuracy of pseudospectral methods. This makes it particularly well-suited for capturing the complex flow physics around airfoils with high fidelity.

Recent advances in DGSEM applications to aerodynamic problems have demonstrated its effectiveness in various flow regimes. Studies on fluid-structure interaction and aerothermodynamics have shown the need for accurate flow field predictions, particularly in the near-wall regions where traditional methods may lack sufficient resolution.

The primary objective of this work is to implement and validate a DGSEM solver for the two-dimensional Navier-Stokes equations, analyze flow characteristics around the NACA 0012 airfoil across different Reynolds numbers, and provide comprehensive validation against experimental data. The study serves as a foundation for future extensions to unsteady phenomena and coupled fluid-structure interaction problems.

# 2  Mathematical Modeling

## 2.1  Governing Equations

The flow around the NACA 0012 airfoil is modeled using the two-dimensional compressible Navier-Stokes equations in conservative form:

∂U/∂t + ∂F/∂x + ∂G/∂y = ∂Fᵥ/∂x + ∂Gᵥ/∂y     (1)

where the conservative variables and inviscid flux vectors are defined as:

U = [ρ, ρu, ρv, ρE]ᵀ,  F = [ρu, ρu² + p, ρuv, ρuH]ᵀ,  G = [ρv, ρuv, ρv² + p, ρvH]ᵀ     (2) and the viscous flux vectors are:

Fᵥ = [0, τₓₓ, τₓᵧ, uτₓₓ + vτₓᵧ + k∂T/∂x]ᵀ

Gᵥ = [0, τₓᵧ, τᵧᵧ, uτₓᵧ + vτᵧᵧ + k∂T/∂y]ᵀ     (3) Here, ρ is the density, u and v are the velocity components in the x and y directions, p is the pressure, E is the specific total energy, H = E + p/ρ is the specific total enthalpy, τᵢⱼ are the viscous stress components, k is the thermal conductivity, and T is the temperature.

The viscous stress components for a Newtonian fluid are:

τₓₓ = μ(2∂u/∂x - (2/3)∇·𝐮)     (4) τᵧᵧ = μ(2∂v/∂y - (2/3)∇·𝐮)     (5) τₓᵧ = μ(∂u/∂y + ∂v/∂x)     (6)

where μ is the dynamic viscosity and ∇·𝐮 = ∂u/∂x + ∂v/∂y is the velocity divergence.

The system is closed by the ideal gas equation of state:

p = (γ - 1)ρ(E - (1/2)(u² + v²))     (7)

where γ = 1.4 is the ratio of specific heats for air.

For low Mach number flows approaching the incompressible limit, the density variations become negligible, and the system reduces to the incompressible Navier-Stokes equations:

∇·𝐮 = 0     (8)

∂𝐮/∂t + (𝐮·∇)𝐮 = -(1/ρ)∇p + ν∇²𝐮     (9)

where ν = μ/ρ is the kinematic viscosity and ∇² = ∂²/∂x² + ∂²/∂y² is the Laplacian operator.

The Reynolds number, defined as Re = ρU∞L/μ, characterizes the relative importance of inertial to viscous forces and will be used to analyze the flow behavior across different regimes.

## 2.2  DGSEM Formulation

### 2.2.1  Weak Formulation

The Discontinuous Galerkin Spectral Element Method (DGSEM) begins with the weak formulation of the Navier-Stokes equations. For each element Ωₑ, we multiply by test functions φ and integrate:

∫[Ωₑ] φ(∂U/∂t)dΩ + ∫[Ωₑ] φ(∂F/∂x + ∂G/∂y)dΩ = ∫[Ωₑ] φ(∂Fᵥ/∂x + ∂Gᵥ/∂y)dΩ     (10) Applying integration by parts to the flux terms:

∫[Ωₑ] φ(∂U/∂t)dΩ - ∫[Ωₑ] (∂φ/∂x·F + ∂φ/∂y·G)dΩ + ∫[∂Ωₑ] φ(Fnₓ + Gnᵧ)dΓ = ∫[Ωₑ] φ(∂Fᵥ/∂x + ∂Gᵥ/∂y)dΩ     (11)

where 𝐧 = (nₓ, nᵧ) is the outward unit normal vector to the element boundary.

For the viscous terms, we apply integration by parts twice to obtain the symmetric interior penalty formulation:

∫[Ωₑ] φ(∂Fᵥ/∂x)dΩ = -∫[Ωₑ] (∂φ/∂x)Fᵥ dΩ + ∫[∂Ωₑ] φFᵥnₓ dΓ     (12)

### 2.2.2  Basis Functions and Quadrature

The DGSEM employs Lagrange interpolating polynomials based on Legendre-GaussLobatto (LGL) points. For polynomial degree N, the basis functions are:

ℓⱼ(ξ) = ∏[k=0 to N, k≠j] (ξ - ξₖ)/(ξⱼ - ξₖ),  j = 0, 1, ..., N     (13)

where ξⱼ are the LGL points on the reference interval [-1, 1], given by:

ξ₀ = -1,  ξₙ = 1,  L'ₙ(ξⱼ) = 0  for j = 1, ..., N-1     (14)

and Lₙ(ξ) is the N-th Legendre polynomial.

The approximate solution within each element is represented as:

Uₕ(ξ, η, t) = Σ[i=0 to N] Σ[j=0 to N] Uᵢⱼ(t)ℓᵢ(ξ)ℓⱼ(η)     (15)

### 2.2.3  Numerical Fluxes

At element interfaces, numerical fluxes are required to couple the discontinuous elements. For the inviscid terms, we employ the Roe approximate Riemann solver, which provides accurate resolution of discontinuities:

F*ₙ = (1/2)[Fₙ(U⁻) + Fₙ(U⁺)] - (1/2)|Ãₙ|(U⁺ - U⁻)     (16)

where Fₙ = F·𝐧 = Fnₓ + Gnᵧ is the normal flux, U⁻ and U⁺ represent the interior and exterior traces, and |Ãₙ| is the absolute value of the Roeaveraged flux Jacobian (matrix with absolute eigenvalues).

The Roe-averaged variables are computed as:

ρ̃ = √(ρ⁻ρ⁺)     (17)

ũ = (√ρ⁻·u⁻ + √ρ⁺·u⁺)/(√ρ⁻ + √ρ⁺)     (18)

ṽ = (√ρ⁻·v⁻ + √ρ⁺·v⁺)/(√ρ⁻ + √ρ⁺)     (19)

H̃ = (√ρ⁻·H⁻ + √ρ⁺·H⁺)/(√ρ⁻ + √ρ⁺)     (20)

The eigenvalues of the normal flux Jacobian are:

λ₁ = Ṽₙ - c̃,  λ₂ = λ₃ = Ṽₙ,  λ₄ = Ṽₙ + c̃     (21)

where Ṽₙ = ũnₓ + ṽnᵧ is the normal velocity and c̃ = √[(γ-1)(H̃ - (1/2)(ũ² + ṽ²))] is the speed of sound.

For enhanced stability in low Mach number flows, we also implement the Lax-Friedrichs flux as an alternative:

F*ₗғ = (1/2)[Fₙ(U⁻) + Fₙ(U⁺)] - (α/2)(U⁺ - U⁻)     (22)

where α = max(|Ṽₙ| + c)̃ is the maximum wave speed.

For the viscous terms, we use the BR2 (Bassi-Rebay 2) scheme to ensure stability and accuracy:

F*ᵥ,ₙ = (1/2)[Fᵥ,ₙ(U⁻) + Fᵥ,ₙ(U⁺)] + (η/h)(U⁺ - U⁻)     (23)

where η is a penalty parameter and h is the element size.

## 2.3  Boundary Conditions

### 2.3.1  Far-field Boundary

At the far-field boundary, we impose characteristic-based boundary conditions:

- = U∞ cos(α)     (24)

- = U∞ sin(α)     (25)

- p = p∞     (26)

T = T∞     (27)

where α is the angle of attack and U∞, p∞, T∞ are the free-stream conditions.

### 2.3.2  Airfoil Surface

On the NACA 0012 surface, we enforce the no-slip boundary condition for viscous flow:

- = 0     (28)

- = 0     (29)

# ∂T/∂n = 0  (adiabatic wall)     (30)

where 𝐧 is the outward unit normal to the airfoil surface.

This boundary condition is implemented through the numerical flux by setting the velocity components to zero at the wall. For the adiabatic wall condition, the temperature gradient normal to the wall is set to zero, which is enforced in the energy equation through appropriate modification of the viscous flux at the wall boundary.

## 2.4  NACA 0012 Geometry

The NACA 0012 airfoil is defined by the thickness distribution:

yₜ = (t/0.2)[0.2969√x - 0.1260x - 0.3516x² + 0.2843x³ - 0.1036x⁴]     (31)

where t = 0.12 is the maximum thickness-to-chord ratio, and x is normalized by the chord length.

The upper and lower surfaces of the airfoil are given by:

yᵤₚₚₑᵣ = +yₜ     (32)

yₗₒwₑᵣ = -yₜ     (33)

## 2.5  Time Integration

### 2.5.1  Explicit Runge-Kutta Scheme

For time advancement of the semi-discrete DGSEM system, we employ the classical fourth-order Runge-Kutta (RK4) method:

k₁ = -M⁻¹R(Uⁿ)     (34)

k₂ = -M⁻¹R(Uⁿ + (Δt/2)k₁)    (35) k₃ = -M⁻¹R(Uⁿ + (Δt/2)k₂)    (36)

k₄ = -M⁻¹R(Uⁿ + Δtk₃)     (37)

Uⁿ⁺¹ = Uⁿ + (Δt/6)(k₁ + 2k₂ + 2k₃ + k₄)     (38)

where M is the mass matrix, R(U) represents the spatial residual (negative of the righthand side), and Δt is the time step.

For DGSEM with Legendre-Gauss-Lobatto quadrature, the mass matrix is diagonal:

Mᵢⱼ = wᵢwⱼJᵢⱼδᵢⱼ     (39)

where wᵢ are the LGL quadrature weights and Jᵢⱼ is the Jacobian of the transformation from physical to reference coordinates.

### 2.5.2  Time Step Restriction

The explicit time integration requires a stability constraint based on the CFL condition:

Δt = CFL · min[e,i,j](hᵢⱼ/(|Vᵢⱼ| + cᵢⱼ))     (40)

where CFL ≤ 1 is the Courant-Friedrichs-Lewy number, hᵢⱼ is the local grid spacing, |Vᵢⱼ| is the local velocity magnitude, and cᵢⱼ is the local speed of sound.

For polynomial degree N = 3, the theoretical CFL limit is approximately CFL ≤ 1/7 ≈

0.14 for optimal stability, though practical values of CFL = 0.1 are typically used.

### 2.5.3  Steady-State Convergence

For steady-state solutions, we monitor the L2 norm of the temporal residual:

Rρ = √[(1/Nₐₒ𝒻)Σ[i=1 to Nₐₒ𝒻](∂ρᵢ/∂t)²]     (41)

Convergence is achieved when Rρ < 10⁻¹² or when the residual drops by at least 8 orders of magnitude from its initial value.

## 2.6  Polynomial Degree and Quadrature Details

For polynomial degree N = 3, we have N+1 = 4 LGL points  on the reference interval [-1, 1]:

ξ = [-1, -√(1/5), √(1/5), 1]ᵀ  (indices 0, 1, 2, 3)     (42)

The corresponding quadrature weights are:

w = [1/6, 5/6, 5/6, 1/6]ᵀ     (43)

The derivative matrix D, which relates nodal values to their derivatives, is approximately:

D ≈ [[-6.00, 8.09, -2.24, 0.50],
 [-1.12, 0.00, 1.54, -0.28],
 [0.28, -1.54, 0.00, 1.12], 
[-0.50, 2.24, -8.09, 6.00]]     (44)

This derivative matrix enables efficient computation of spatial derivatives at the quadrature points through matrix-vector multiplication.

## 2.7  Aerodynamic Coefficients

For viscous flow, the aerodynamic coefficients are computed by integrating both pressure and viscous forces around the airfoil surface:

Cₗ = (1/(½ρ∞U²∞c)) ∮[airfoil] (pnᵧ + τₓᵧnₓ + τᵧᵧnᵧ)ds     (45)

Cₐ = (1/(½ρ∞U²∞c)) ∮[airfoil] (pnₓ + τₓₓnₓ + τₓᵧnᵧ)ds     (46)

where 𝐧 is the outward unit normal vector, c is the chord length, and τᵢⱼ are the viscous stress components.

The moment coefficient about the quarter-chord point is:

Cₘ = (1/(½ρ∞U²∞c²)) ∮[airfoil] [(x-0.25c)(pnᵧ + τₓᵧnₓ + τᵧᵧnᵧ) - y(pnₓ + τₓₓnₓ + τₓᵧnᵧ)]ds     (47)

For theoretical inviscid flow around a symmetric airfoil at zero angle of attack, we expect:

- • Cₗ = 0 (no circulation)

- • Cₐ = 0 (d'Alembert's paradox)

- • Cₘ = 0 (symmetric loading)

However, for viscous flow, drag is non-zero due to viscous effects, and small lift and moment may arise from numerical dissipation and discretization errors.

## 2.8  Verification Cases

Before analyzing the NACA 0012, we will verify the DGSEM implementation using analytical solutions:

### 2.8.1  Isentropic Vortex

The two-dimensional isentropic vortex provides an exact solution to the Euler equations with convection velocity U = 1 :

ρ(x,y,t) = [1 - ((γ-1)β²)/(8γπ²)e^(1-r²)]^(1/(γ-1))     (48) u(x,y,t) = 1 - (β/2π)(y - yc)e^((1-r²)/2)     (49)

v(x,y,t) = (β/2π)(x - xc)e^((1-r²)/2)     (50)

p(x,y,t) = ρ^γ     (51)

where r² = (x - xc - t)² + (y - yc)², β = 5 is the vortex strength, and (xc, yc) is the initial vortex center. The vortex convects with unit velocity in the x-direction.

### 2.8.2  Method of Manufactured Solutions

We also employ manufactured solutions of the form:

ρₑₓₐcₜ = 1 + 0.1sin(πx)cos(πy)     (52)

uₑₓₐcₜ = sin(πx)cos(πy)     (53)

vₑₓₐcₜ = -cos(πx)sin(πy)     (54)

# pₑₓₐcₜ = 1 + 0.1cos(πx)sin(πy)     (55)

The velocity field satisfies ∇·𝐮 = 0 (incompressible). The corresponding source terms are computed symbolically and added to the right-hand side of the governing equations to verify spatial accuracy for both inviscid and viscous terms. This tests the solver's ability to handle arbitrary smooth solutions despite solving the compressible equations.

## References

[1] N. Gregory and C. L. O'Reilly, "Low-Speed Aerodynamic Characteristics of NACA 0012 Aerofoil Section, including the Effects of Upper-Surface Roughness Simulating Hoar Frost," Reports and Memoranda No. 3726, Aeronautical Research Council, 1970.